WOLFWORKS LABS LLC

# Engineering Toolkit

Project Scope & Development Plan

Flask-based engineering, automation, server, data-format, and networking utility suite

| Framework | Flask / Python |
| --- | --- |
| Sprint Length | Two weeks |
| Primary Goal | Build a functional, modular engineering toolkit with dynamic server-generated pages |
| Stretch Goals | Database integration, multiple dynamic pages, and Python library integration |

Designed as an expandable foundation,  for practical use in education instruction rather than a one-off assignment.

# 1. Project Overview

WolfWorks Engineering Toolkit is a modular web application that combines practical engineering and IT utilities into a single consistent interface. The application will be built with Flask and will dynamically generate HTML using Jinja templates. Users will provide data through forms, the server will validate and process that data in Python, and the resulting calculations, comparisons, configurations, and error messages will be rendered back into the web interface.

The project intentionally exceeds the minimum class requirements while remaining achievable within a two-week sprint. Each module is useful independently, allowing the project to remain functional even if optional features are deferred.

## Sprint Goal

The goal of this sprint is to develop a functional Flask-based engineering toolkit that provides several interactive tools for electrical calculations, PLC I/O configuration, server power planning, file format comparison, and network/subnet calculations. The application will validate user input, dynamically generate results, use a database to store applicable data, and integrate Python libraries for additional functionality. The finished application should provide a consistent and expandable foundation for adding additional engineering and IT tools in the future.

## Project Objectives

Create a server-side web application using Flask and Python.

Provide a unified interface for multiple engineering and IT tool categories.

Accept, validate, and process user input safely and predictably.

Generate HTML content dynamically based on calculations, database records, and user selections.

Use a maintainable modular architecture so additional tools can be added later.

Complete all required assignment criteria and intentionally implement all three stretch challenges.

# 2. Scope

## In Scope for This Sprint

Shared Flask application shell, navigation, styling, and reusable Jinja base template.

Electrical engineering calculators.

PLC I/O configuration and signal-scaling tools.

Server and rack power-planning tools.

Database-backed file format explorer and comparison tool.

IPv4 and subnet-planning tools.

Server-side input validation and useful error messages.

SQLite persistence through SQLAlchemy where persistent data is appropriate.

Testing of calculation functions and important boundary conditions.

Responsive, consistent presentation suitable for a class demonstration.

## Out of Scope / Future Expansion

User accounts, authentication, roles, or cloud synchronization.

Real-time collaboration or multi-user editing.

Production payment, commerce, or subscription features.

Full enterprise PLC programming or hardware communication.

Automatic hardware discovery or live server monitoring.

A complete replacement for dedicated network-design or electrical-CAD software.

Deployment infrastructure beyond what is needed to demonstrate the application.

# 3. Technology Stack

| Technology | Purpose |
| --- | --- |
| Python | Primary programming language and calculation logic |
| Flask | Web framework, request routing, and application structure |
| Jinja2 | Server-side dynamic HTML templating |
| Flask Blueprints | Modular organization of the five tool categories |
| SQLAlchemy | Object-relational mapping and database access |
| SQLite | Lightweight local relational database |
| HTML / CSS | User interface, forms, static assets, and responsive presentation |
| ipaddress | Python networking library used for IPv4/CIDR processing |
| pytest or unittest | Automated testing of calculation and validation logic |

# 4. Application Architecture

The project will use Flask Blueprints so each major tool category remains independent while sharing one application, database configuration, visual design, and base template. Calculation logic will be kept separate from route handlers so it can be tested without requiring a browser request.

Browser Request
      |
      v
Flask Route / Blueprint
      |
      +--> Validate Input
      |
      +--> calculations.py / Python library
      |
      +--> SQLAlchemy / SQLite (when needed)
      |
      v
Jinja Template
      |
      v
Dynamically Generated HTML Response

## Planned Project Structure

wolfworks_toolkit/
|-- run.py
|-- requirements.txt
|-- tests/
`-- wolfworks/
    |-- __init__.py
    |-- extensions.py
    |-- models.py
    |-- core/
    |-- electrical/
    |-- plc/
    |-- servertools/
    |-- fileformats/
    |-- networking/
    |-- templates/
    `-- static/
        |-- css/
        |-- js/
        `-- images/

# 5. Functional Modules

## 5.1 Electrical Engineering Tools

A collection of reusable calculators for common electrical and controls calculations.

Ohm's Law: calculate voltage, current, resistance, and power from known values.

Voltage Divider: calculate output voltage, circuit current, voltage drops, and resistor power.

Series and Parallel Resistance: calculate equivalent resistance from multiple values.

ADC Calculator: convert ADC counts to voltage and voltage to ADC counts using configurable resolution and reference voltage.

DAC Calculator: perform corresponding digital-to-analog conversions.

Industrial Analog Scaling: map 0-10 V, 1-5 V, 4-20 mA, or custom electrical ranges to engineering units.

Representative validation includes zero-division protection, positive resistance requirements, ADC count limits, numeric validation, and electrical-range bounds.

## 5.2 PLC I/O Configurator

A configuration tool for building and reviewing a PLC I/O list.

Create I/O points with tag name, description, signal type, channel, electrical range, engineering range, and units.

Support common AI, AO, DI, and DO classifications.

Calculate analog scaling between electrical and engineering units.

Display a dynamically generated I/O table.

Persist applicable configurations or I/O records in SQLite.

Detect invalid ranges and, where implemented, duplicate or conflicting channel assignments.

## 5.3 Server / Rack Planner

A planning tool for estimating server, workstation, and rack electrical requirements.

Add component quantities and estimated wattages.

Calculate total system load and configurable design headroom.

Calculate current at common supply voltages.

Estimate energy consumption and operating cost from user-provided electricity rates.

Convert electrical load to approximate heat output in BTU/hr.

Provide circuit-utilization and capacity information based on user inputs.

Optional extension: simplified UPS runtime estimates when sufficient battery data is supplied.

## 5.4 File Format Explorer

A database-backed reference for structured data, configuration, resource, and database file formats.

Initial formats may include JSON, JSONC, XML, YAML/YML, CSV, TSV, TXT, INI, TOML, NBT, REG, SQLite, MDB, XLS, XLSX, and DAT.

Store attributes such as extensions, category, text/binary representation, human readability, hierarchy, comment support, common uses, advantages, and limitations.

Generate individual format pages from database records.

Allow the user to select two formats and dynamically generate a side-by-side comparison.

## 5.5 Network / Subnet Toolkit

Networking utilities built around Python's ipaddress library.

IPv4/CIDR analyzer: network, prefix, subnet mask, broadcast address, address count, and host range.

Subnet calculator: divide a network into a requested number of subnets.

Host requirement calculator: determine a suitable subnet size for a requested host count.

Validate malformed addresses, prefixes, and impossible requests.

Optional extension: basic VLAN/address-plan generation.

# 6. Database Plan

SQLite will be integrated through SQLAlchemy. The database is not required for every calculator; it will be used where persistence or structured reference data adds value.

| Model | Purpose |
| --- | --- |
| FileFormat | Stores file-format metadata used by the explorer and comparison pages. |
| PLCConfiguration | Represents a saved PLC I/O configuration or project. |
| IOPoint | Stores individual I/O points associated with a PLC configuration. |

# 7. Assignment Requirement Coverage

| Requirement | Implementation |
| --- | --- |
| Generate at least one HTML page | Flask/Jinja will generate the homepage and every tool/result page dynamically. |
| Include pictures and/or CSS stored with app code | Shared CSS, icons/images, favicon, and other static assets will be stored under the application static directory. |
| Accept user input | Calculators and configurators use forms, selectors, buttons, and numeric/text inputs. |
| Perform error checking | Server-side validation will reject invalid numeric ranges, malformed IP addresses, impossible calculations, and incomplete data. |
| Modify generated HTML based on user input | Submitted values determine calculated results, comparison tables, I/O lists, validation messages, and network information. |
| Stretch: database | SQLAlchemy + SQLite support the file-format explorer and applicable PLC configuration data. |
| Stretch: third dynamic page | The application contains numerous dynamically generated tool and result pages. |
| Stretch: Python/JavaScript library | Python's ipaddress library performs networking calculations; additional libraries may be incorporated where useful. |

# 8. Minimum Functional Base (MFB)

The minimum functional base is deliberately small so the project becomes demonstrably functional early in the sprint. Once this milestone works, every additional module builds on a proven request/validation/calculation/template pattern.

Flask application starts successfully and serves a shared homepage.

Base Jinja template, navigation, CSS, and static assets load correctly.

Electrical Blueprint is registered and reachable.

Ohm's Law form accepts user input.

Server-side validation detects invalid input.

Python calculation logic produces a result.

The result is rendered dynamically into HTML.

At least one automated test verifies the calculation logic.

Target: achieve the minimum functional base by approximately Day 3. At that point, the core class requirements are already represented and later work expands capability rather than rescuing an incomplete application.

# 9. Development Plan

| Day | Focus | Deliverable |
| --- | --- | --- |
| 1 | Foundation | Repository, virtual environment, Flask installation, application factory, Blueprints, initial project structure. |
| 2 | Shared UI | Base template, homepage, navigation, CSS/static assets, common form/result styling. |
| 3 | Electrical I | Ohm's Law and voltage-divider tools; establish reusable validation/calculation pattern. |
| 4 | Electrical II | ADC/DAC, resistance, and industrial analog scaling tools. |
| 5 | PLC I | PLC I/O configurator forms, dynamic I/O table, scaling logic. |
| 6 | PLC II / Database | SQLAlchemy/SQLite integration and persistence for applicable PLC data. |
| 7 | Server Tools | Power, current, headroom, energy, cost, and heat calculations. |
| 8 | File Formats I | Database model/data population and format-detail pages. |
| 9 | File Formats II | Dynamic format comparison interface and validation. |
| 10 | Networking I | IPv4/CIDR analyzer using ipaddress. |
| 11 | Networking II | Subnet splitting and host-requirement tools; optional VLAN planner if time permits. |
| 12 | Validation | Boundary conditions, error messages, invalid data, and cross-module consistency. |
| 13 | Testing / Polish | Automated tests, responsive layout, formatting, accessibility labels, UI cleanup. |
| 14 | Release | Documentation, final testing, cleanup, demonstration preparation, and submission. |

# 10. Validation and Testing Strategy

Keep calculation logic in dedicated Python functions so it can be tested independently of Flask routes.

Test normal values, boundaries, invalid types, missing values, zero values where prohibited, and out-of-range inputs.

Verify server-side validation even when browser-side HTML validation is bypassed.

Test important network edge cases such as invalid octets, invalid prefixes, small subnets, and impossible subnet requests.

Test database-backed pages with known records and comparison selections.

Perform browser-based integration testing of each form from input through rendered result.

## Representative Test Cases

| Area | Example |
| --- | --- |
| Electrical | 12 V and 2 A -> 6 ohms; reject invalid zero divisor or negative resistance where inappropriate. |
| ADC | 12-bit ADC accepts 0-4095; reject a count such as 9000. |
| Analog | 4-20 mA mapped to 0-100% gives 12 mA -> 50%. |
| PLC | Reject invalid engineering/electrical ranges and conflicting channels when conflict detection is enabled. |
| Server | Verify quantity x wattage totals, voltage/current relationship, and user-entered cost calculations. |
| Network | Reject 192.168.400.12; correctly analyze valid CIDR input. |

# 11. Priorities and Scope Control

The project is intentionally modular. Core functionality is prioritized over optional polish so the application remains shippable throughout the sprint.

| Priority | Meaning |
| --- | --- |
| P0 - Required | Flask application, shared UI, dynamic forms/results, input validation, and at least one complete calculator. |
| P1 - Planned | All five major modules, SQLAlchemy/SQLite integration, ipaddress integration, and representative automated tests. |
| P2 - Enhancement | Saved PLC projects, advanced server calculations, richer file-format data, VLAN planner, additional calculators. |
| P3 - Future | Authentication, cloud persistence, live hardware integration, APIs, exports, or production-scale deployment. |

# 12. Definition of Done

The Flask application runs reliably in the intended development environment.

All completed tools are reachable through consistent navigation.

Forms accept and validate user input server-side.

Calculated or database-derived results are rendered dynamically in HTML.

Static CSS/images are packaged with the application.

SQLite/SQLAlchemy database functionality is demonstrated.

The networking module demonstrates Python library integration.

Core calculation functions have representative automated tests.

The application handles common invalid inputs without crashing.

The repository contains clear setup/run instructions and required dependencies.

The application is ready for a live class demonstration.

# 13. Future Expansion

The toolkit is intended to remain useful after the class sprint. New calculators can be added as Blueprint routes and calculation modules without redesigning the application.

Additional electrical calculators such as RC time constants, power-factor tools, conductor voltage drop, and resistor color codes.

PLC tag export/import helpers and additional industrial signal standards.

Rack-unit planning, PSU efficiency curves, and more detailed UPS/battery modeling.

Additional file formats and searchable/filterable format metadata.

IPv6 tools, VLSM planning, VLAN documentation, and address-plan export.

REST API endpoints for selected calculations.

Export of results to CSV, JSON, or printable reports.

# 14. Success Criteria

The sprint will be considered successful when WolfWorks Engineering Toolkit operates as a cohesive Flask application rather than a collection of disconnected demos. A user should be able to navigate between modules, enter valid or invalid data, receive clear feedback, and obtain dynamically generated engineering or IT results. The codebase should be organized well enough that another tool can be added without restructuring the entire project.